Table S9. Canonical Sentinel-1 dual-polarisation backscatter signatures for the three inundation mechanisms exploited by the Module 02 saline-flood classifier.

| Mechanism | Onset DOY | Onset rate (d) | VH base (dB) | VH min (dB) | ΔVH (dB) | VV base (dB) | VV min (dB) | ΔVV (dB) | CR base (dB) | CR min (dB) | ΔCR (dB) | VH recov. (d) | VV recov. (d) | CR recov. (d) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transplanting flood (agronomic) | 188 | 12 | -14.0 | -20.5 | -6.5 | -9.0 | -13.5 | -4.5 | -5.0 | -7.0 | -2.0 | 35 | 50 | 42 |
| Saline storm-surge (cyclonic) | 123 | 1 | -12.0 | -22.5 | -10.5 | -8.5 | -15.0 | -6.5 | -3.5 | -7.5 | -4.0 | 21 | 28 | 24 |
| Freshwater rainfall (Bulbul-class) | 313 | 2 | -13.5 | -16.5 | -3.0 | -8.5 | -10.0 | -1.5 | -5.0 | -6.5 | -1.5 | 10 | 8 | 9 |

VH and VV are calibrated dual-polarisation backscatter in decibels (Sentinel-1 IW GRD). CR = VH − VV (dB) is the cross-ratio. Onset rate is the time from baseline to trough minimum. Recovery is the e-folding time from trough to within 1 dB of baseline. Values are canonical idealisations calibrated against Hoshikawa et al. (2023), Wali et al. (2020), Filipponi (2019), Konkathi et al. (2024), and Lee & Pottier (2009); they specify the feature space exploited by the Module 02 random-forest classifier (§3.3) and are not produced by direct measurement.